# ARDU-8051 EDU-PRO

## Student Laboratory Manual

### Course Context

This laboratory manual is designed for undergraduate microcontroller courses where the 8051 architecture is taught as a foundational platform. The ARDU-8051 EDU-PRO board combines traditional 8051 learning with Arduino-style hardware convenience and an integrated USBasp programmer.

## Lab 0: Getting Started & Board Familiarization

### Objectives

- Identify major sections of the ARDU-8051 EDU-PRO board

- Understand power, clock, and programming interfaces

- Prepare the board for first-time use

### Required Equipment

- ARDU-8051 EDU-PRO board

- USB cable (Type-B or Type-C)

- PC with USBasp driver installed

### Board Sections to Identify

- AT89S52 (8051 microcontroller)

- On-board USBasp programmer (ATmega8A)

- USB connector (Type-B / Type-C)

- Power regulation section

- Port headers (P0, P1, P2, P3)

Observation: Students should sketch a block-level diagram of the board in their lab notebook.

## Lab 1: Blink LED Using 8051 (Basic GPIO Test)

### Objective

To verify correct operation of the ARDU-8051 EDU-PRO board by executing a Blink LED program using Port-1 GPIO.

### Learning Outcomes

After completing this experiment, students will be able to: - Configure an 8051 I/O pin as output - Generate software delay loops - Program an 8051 microcontroller using USBasp

### Theory (Brief)

In the 8051 microcontroller, each port pin can be configured as an input or output. Writing logic 0 or 1 to a port pin controls the voltage level, which can be used to drive external devices such as LEDs. A delay loop is implemented using nested for loops to generate approximate time delays.

### Hardware Required

- ARDU-8051 EDU-PRO board

- USB cable

- LED (external, if on-board LED is not used)

- 330 Ω resistor

### Circuit Connection

Option A: On-board LED - LED is pre-connected to P1.0

Option B: External LED

P1.0  → 330 Ω → LED → GND

Note: The LED is assumed to be active-low.

### Software Tools

- Keil µVision (recommended)

- USBasp programming utility (ProgISP / Extreme Burner / avrdude)

### Program Code (Blink LED)

#include <reg52.h>

sbit LED = P1^0;

void delay_ms(unsigned int ms)
{
    unsigned int i, j;
    for(i = 0; i < ms; i++)
        for(j = 0; j < 1275; j++);
}

void main(void)
{
    while(1)
    {
        LED = 0;    // LED ON
        delay_ms(500);
        LED = 1;    // LED OFF
        delay_ms(500);
    }
}

### Procedure

- Connect the ARDU-8051 EDU-PRO board to the PC using a USB cable.

- Verify that the power LED on the board is ON.

- Open Keil µVision and create a new project.

- Select AT89S52 as the target device.

- Add the Blink LED source code.

- Enable Create HEX File in project options.

- Build the project to generate the HEX file.

- Open the USBasp programming software.

- Select programmer as USBasp and device as AT89S52.

- Load the HEX file and program the device.

- Observe the LED behavior.

### Expected Result

- The LED connected to P1.0 turns ON for approximately 0.5 seconds

- The LED turns OFF for approximately 0.5 seconds

- The process repeats continuously

### Troubleshooting

| Problem | Possible Cause | Solution |
| --- | --- | --- |
| LED not blinking | Wrong port pin | Verify LED connection |
| Programming fails | USBasp not detected | Check driver installation |
| No HEX file | HEX generation disabled | Enable HEX option |
| Board not powered | USB cable issue | Try different cable/port |

### Viva / Review Questions

- Why is Port-1 preferred for simple LED experiments in 8051?

- What determines the delay time in the software loop?

- Why is USBasp suitable for educational use?

- What happens if the crystal frequency is changed?

### Lab Assignment

- Modify the program to blink the LED at a 1-second interval.

- Connect an LED to another Port-1 pin and blink alternately.

- Rewrite the delay using Timer-0 instead of software loops.

## Notes for Instructors

- This experiment serves as a hardware health check.

- Subsequent labs may include ADC0804 interfacing, timers, interrupts, LCD, and UART.

- Students should retain this board for the entire semester.

End of Experiment